S.A.F.E.

Swarm Autonomous Field Explorer

DEPLOYMENT, OPERATION & DEBUG REFERENCE

ROBOFEST-GUJARAT 5.0  ·  Senior Category  ·  Aerial Robotics

Version 1.0

| master 10.42.0.1 | slave_1 10.42.0.11 | slave_2 10.42.0.12 | slave_3 10.42.0.13 |
| --- | --- | --- | --- |

# 1  Folder Structure

Every file the swarm needs lives under a single project root. The structure below must be identical on all four Pis — only the DRONE_ID environment variable differentiates them at runtime.

safe_swarm/                          ← project root (same on all 4 Pis)

│

├── main_orchestrator.py             ← slave mission loop (runs on slave_1/2/3)

├── hover_test.py                    ← pre-flight hover & formation test (master only)

├── sim_harness.py                   ← ground-level comms integration test (laptop/master)

│

├── tcp_commander.py                 ← master: sends GOTO/PAUSE/LAND to slaves

├── tcp_channel.py                   ← shared: TCPCommandServer + TCPCommandClient classes

├── udp_channel.py                   ← shared: UDPSender + UDPReceiver classes

├── swarm_state.py                   ← shared: single source of truth (STATE singleton)

│

├── 02_vision_filter.py              ← slave: thermal blob detection

├── 03_persistence.py                ← slave: mine persistence checker

├── 04_coordinate_math.py            ← slave: pixel → GPS conversion

├── 05_map_verifier.py               ← slave: deduplication before broadcast

├── 08_comms_link.py                 ← slave: DroneTunnel (mine TCP reports to master)

│

├── udp_telemetry.py                 ← master: UDP server + SwarmState writer

├── app.py                           ← master: Flask ground server + voice API

│

├── config/

│   ├── fpn_pattern.npy              ← thermal noise calibration (from 00_preflight_calib.py)

│   └── origin_state.json            ← GPS+yaw lock at mission start

│

├── logs/                            ← auto-created at runtime

│   ├── slave_1.log

│   ├── slave_2.log

│   └── slave_3.log

│

└── bin/

└── mlx_stdout                   ← compiled C++ binary: reads MLX90640 → stdout

⚠ Files that must exist before flight

config/fpn_pattern.npy — run 00_preflight_calib.py once with sensor warm to generate

config/origin_state.json — created automatically on first GPS lock at mission start

bin/mlx_stdout — compile from mlx_stdout.cpp; see Section 6 for command

## 1.1  File Roles at a Glance

| File | Role |
| --- | --- |
| main_orchestrator.py | Slave mission brain. Starts camera pipe, MAVSDK, UDP sender, TCP server, vision pipeline. Set DRONE_ID env var before running. |
| hover_test.py | Run on master ONLY before any real mission. Arms all 4 drones, checks formation, advances 3m, lands. Safe way to verify the whole stack is wired up. |
| sim_harness.py | No hardware needed. Simulates all 3 slaves as fake TCP/UDP servers on localhost. Proves comms layer works before flying. |
| tcp_commander.py | Master-side: one persistent TCP connection per slave. Collision-safe GOTO dispatch. Voice command fan-out. Per-slave async queues. |
| tcp_channel.py | TCPCommandServer (slave) + TCPCommandClient (master) + Broadcaster. Newline-delimited JSON. ORDER→PROCESS→RESULT pattern. |
| udp_channel.py | UDPSender (slave, 5 Hz) + UDPReceiver (master, single port). Stale-packet rejection by seq number. Mine persistence tracker. |
| swarm_state.py | SwarmState singleton STATE. All drone positions, mines, mission config. Only place position data is written. Lock-protected async helpers. |
| 08_comms_link.py | DroneTunnel: slave sends confirmed mine GPS + thermal image to master port 5000 over TCP with 4-byte length-prefix framing. |
| udp_telemetry.py | Master UDP server coroutine. Calls STATE.update_drone() per packet. Forwards to app.py over HTTP POST. |
| app.py | Flask server for ground station phone. /api/drone_update (POST), /api/pending_commands (GET). Bridges voice commands into tcp_commander. |

# 2  Network Layout

The master Pi acts as a Wi-Fi hotspot via dnsmasq. All slaves connect to it. IPs are assigned by MAC via dnsmasq config — they never change.

| Node | Role |
| --- | --- |
| master 10.42.0.1 | Pi hotspot — runs tcp_commander, udp_telemetry, app.py, hover_test |
| slave_1 10.42.0.11 | Connects to master hotspot — runs main_orchestrator DRONE_ID=slave_1 |
| slave_2 10.42.0.12 | Connects to master hotspot — runs main_orchestrator DRONE_ID=slave_2 |
| slave_3 10.42.0.13 | Connects to master hotspot — runs main_orchestrator DRONE_ID=slave_3 |

## 2.1  Ports

| Port | Purpose |
| --- | --- |
| UDP 14550 | Slave → Master. 5 Hz telemetry (lat/lng/alt/heading/armed/sensor). UDPSender→UDPReceiver. |
| TCP 14560 | Master → Slave. Command channel (GOTO/PAUSE/RESUME/LAND/ARM_TAKEOFF). One conn per slave, persistent. |
| TCP 5000 | Slave → Master. Mine anomaly report via DroneTunnel. 4-byte length prefix + JSON body. |
| TCP 443 | Master app.py HTTPS. /api/drone_update /api/pending_commands. Self-signed cert. |
| UDP 14540 | FC → MAVProxy → MAVSDK. Each drone local only. Not cross-Pi. |

## 2.2  Connection Direction Summary

slave_1 ──UDP 14550──► master   (telemetry, continuous)

slave_2 ──UDP 14550──► master   (same port, drone_id field demuxes)

slave_3 ──UDP 14550──► master

master ──TCP 14560──► slave_1   (commands, persistent connection)

master ──TCP 14560──► slave_2

master ──TCP 14560──► slave_3

slave_1 ──TCP 5000──► master    (mine reports, per-event connection)

slave_2 ──TCP 5000──► master

slave_3 ──TCP 5000──► master

phone browser ──HTTPS 443──► master app.py   (ground station UI)

# 3  Startup Sequence

Order matters. Start MAVProxy first (it opens the serial port), then MAVSDK-consuming Python scripts. Always start slaves before master so the master's TCP connection attempts find live servers.

## 3.1  On Each Slave Pi  (drone1, drone2, drone3)

Run these two commands in order. Use tmux or two SSH sessions.

# ── Terminal 1: MAVLink proxy ──────────────────────────────────────────

mavproxy.py \

--master=/dev/ttyAMA0 \

--baudrate=921600 \

--out=udp:0.0.0.0:14540

# Wait until you see:  'Received 1 params'  or  'Got HEARTBEAT'

# If serial port differs, check:  ls /dev/tty*  (may be ttyUSB0, ttyS0)

# ── Terminal 2: Slave orchestrator ─────────────────────────────────────

cd ~/safe_swarm

export DRONE_ID=slave_1          # change to slave_2 / slave_3 on other Pis

export MASTER_IP=10.42.0.1       # always the master hotspot IP

python3 main_orchestrator.py 2>&1 | tee logs/slave_1.log

Expected output on slave startup

[HH:MM:SS][INIT][OK] Vision/logic modules imported

[HH:MM:SS][INIT][OK] udp_channel + tcp_channel imported

[HH:MM:SS][MOVE][OK] FC connected via MAVSDK

[HH:MM:SS][MOVE][OK] GPS fix acquired: lat=XX.XXXXXX lon=YY.YYYYYY

[HH:MM:SS][TCP][OK] TCPCommandServer listening on port 14560

[HH:MM:SS][UDP][OK] Telemetry broadcast started → 10.42.0.1:14550 @ 5 Hz

## 3.2  On the Master Pi

# ── Terminal 1: Master MAVLink proxy (for master drone's own FC) ────────

mavproxy.py \

--master=/dev/ttyAMA0 \

--baudrate=921600 \

--out=udp:0.0.0.0:14540

# ── Terminal 2: Hover test (pre-flight only) ────────────────────────────

cd ~/safe_swarm

python3 hover_test.py

# ── OR Terminal 2: Full mission stack ───────────────────────────────────

cd ~/safe_swarm

python3 app.py &           # ground server (background)

python3 udp_telemetry.py & # UDP receiver (background)

python3 tcp_commander.py   # foreground — connects to slaves, manages mission

## 3.3  Pre-flight Checklist

- Wi-Fi: all 3 slave Pis visible in  arp -a  on master

- Ports free on master:  lsof -i :14550  lsof -i :14560  lsof -i :5000

- MAVProxy running on all 4 Pis — heartbeat visible in terminal

- main_orchestrator.py shows GPS fix on all 3 slaves

- config/fpn_pattern.npy exists on all slave Pis

- Battery > 80% on all drones

- Props clear of personnel and obstructions

- hover_test.py run and all steps PASS before mission

# 4  hover_test.py — Pre-flight Test

Run on the master Pi only. Verifies the full command path: TCP connections, ARM+TAKEOFF, GPS-based formation waypoints, simultaneous movement, and landing — without any mission logic.

## 4.1  What It Tests (in order)

| Step | What Happens |
| --- | --- |
| Step 0 — Connect | TCP connect to each slave on port 14560. Any slave that refuses connection is marked SKIP. Script does not crash — partial test continues. |
| Step 0 — Master FC | MAVSDK connect to master's own FC. On failure, master steps are skipped with a WARN, not an error. |
| Step 1 — ARM_TAKEOFF | Sends ARM_TAKEOFF cmd to all connected slaves simultaneously (threads). Master arms and takes off separately. Waits 8s for climb. |
| Step 2 — Formation | Computes GPS offsets from master's current position. Slaves spread: −1.5m / 0 / +1.5m East. Master positions 1.5m behind slave plane (South). |
| Step 3 — Advance | All slaves GOTO +3m North simultaneously. Master holds. Waits for travel time. |
| Step 4 — Master Follow | Master follows to its new formation position (3m North of start, 1.5m behind slaves). Waits for waypoint confirmation. |
| Step 5 — Land | Slaves land simultaneously. 8s pause. Master lands. |

## 4.2  Formation Geometry

↑ North (forward)

[slave_1]  [slave_2]  [slave_3]

−1.5m E    0m         +1.5m E

↑ 0m N     ↑ 0m N     ↑ 0m N

[master]

0m E,  −1.5m N  (behind slave plane)

After Step 3 (slaves advance +3m):

slaves move to 3m N,  master moves to 1.5m N

1.5m gap maintained throughout

## 4.3  Pass/Fail Output

Every step is recorded. Final summary printed after landing:

────────────────────────────────────────────────────────────

TEST SUMMARY

────────────────────────────────────────────────────────────

✓  [connect                    ] slave_1    (10.42.0.11:14560)

✓  [connect                    ] slave_2    (10.42.0.12:14560)

✗  [connect                    ] slave_3    (10.42.0.13:14560)  ← skipped

✓  [connect                    ] master

✓  [arm_takeoff                ] slave_1

✓  [arm_takeoff                ] slave_2

✓  [arm_takeoff                ] master

✓  [reach_altitude             ] master     final alt=0.97m

...

Result: 11/12 passed  🔴 1 FAILED

## 4.4  TCP Command: ARM_TAKEOFF

hover_test.py sends { "cmd": "ARM_TAKEOFF", "alt": 1.0, "seq": N } over TCP. Your slave's TCPCommandServer in main_orchestrator.py must have a handler for this command. Register it alongside the existing GOTO/PAUSE/RESUME/LAND handlers:

@server.on_command("ARM_TAKEOFF")

def handle_arm_takeoff(cmd: dict) -> dict:

alt = float(cmd.get("alt", 1.0))

asyncio.run_coroutine_threadsafe(

movement.takeoff_to_hover(alt),

asyncio.get_running_loop(),

)

return {"status": "ok"}

⚠ If ARM_TAKEOFF is not registered

Slave will reply: { "ack": "ARM_TAKEOFF", "status": "rejected", "reason": "no handler for 'ARM_TAKEOFF'" }

hover_test.py will record FAIL for that slave's arm_takeoff step but continue.

Fix: add the handler above to main_orchestrator.py start_tcp_command_server().

# 5  sim_harness.py — Integration Test (No Hardware)

Run on a laptop or the master Pi with no drones powered on. Proves the full communication stack — UDP telemetry, TCP commands, mine TCP reports, app.py forwarding, voice bridge — all on localhost.

## 5.1  What It Simulates

| Component | What It Does |
| --- | --- |
| SlaveSimulator ×3 | Fake drones. Send real JSON UDP packets to port 14550. Open TCP command server on 14560/14561/14562. Send fake DroneTunnel mine report to port 5000. |
| FakeState | In-memory SwarmState replacement. No MAVSDK. update_drone() + add_mine() are async but talk to dicts, not databases or hardware. |
| run_fake_app() | Minimal HTTP server on port 8080. Mimics app.py's /api/drone_update (POST) and /api/pending_commands (GET). Pre-loads 'start' voice command. |
| Real udp_telemetry | Actual udp_telemetry.start_udp_server() runs against FakeState. Tests real packet parsing and STATE update path. |
| Real tcp_commander | Actual start_tcp_commander() runs against fake slave TCP servers. Tests real connection + dispatch logic. |

## 5.2  Tests and What PASS Means

| Test | PASS Means |
| --- | --- |
| UDP telemetry path | Master received UDP packets from all 3 fake slaves. Means: UDPSender format is correct, UDPReceiver parsed it, drone_id field matches. |
| STATE population | All 3 drone IDs appear in FakeState after UDP receive. Means: STATE.update_drone() was called and lat/lng stored. |
| Mine TCP path | DroneTunnel anomaly_report from slave_1 arrived at master port 5000 and was added to STATE.mines. Means: 4-byte framing + JSON parse + add_mine() work. |
| app.py mine forward | Mine event POSTed to fake app.py /api/drone_update. Means: udp_telemetry._post_to_ground() ran and HTTP POST succeeded. |
| TCP command path | PAUSE command pushed into STATE.cmd_queues['slave_1'], reached fake slave TCP server, ACK returned. Means: full dispatcher path works. |
| Voice bridge | tcp_commander._voice_bridge polled fake app.py /api/pending_commands and consumed the pre-loaded 'start' command. Means: voice→slave fan-out path works. |

## 5.3  How to Run

cd ~/safe_swarm

pip install flask aiohttp --break-system-packages

python3 sim_harness.py

# All 6 lines should print PASS.

# If any FAIL, see Section 7 for diagnosis.

# 6  Build & Environment Setup

## 6.1  Python Dependencies

# On all 4 Pis:

pip install mavsdk asyncio aiohttp flask numpy --break-system-packages

# Verify MAVSDK server binary exists (mavsdk ships its own):

python3 -c "import mavsdk; print('mavsdk OK')"

## 6.2  Compile the MLX90640 C++ Binary  (slaves only)

# Requires:  libmlx90640-dev  or  bundled vendor headers

g++ -O2 -o bin/mlx_stdout src/mlx_stdout.cpp -lmlx90640

# Verify it streams frames to stdout:

bin/mlx_stdout | xxd | head -4

# Should show non-zero floats (768 floats × 4 bytes = 3072 bytes per frame)

## 6.3  Environment Variables

| Variable | Purpose / Default |
| --- | --- |
| DRONE_ID | slave_1 ¦ slave_2 ¦ slave_3 — determines strip Y offset and UDP sender ID. Not set on master. |
| MASTER_IP | IP of master Pi. Default: 10.42.0.1. Override if network differs. |
| MAVSDK_ADDR | MAVSDK system address. Default: udp://:14540. Override if MAVProxy output port differs. |

## 6.4  MAVProxy Serial Port

The serial port name depends on hardware. Check with:

ls /dev/tty*

# Pixhawk via GPIO UART:  /dev/ttyAMA0  (most Pi setups)

# Pixhawk via USB:         /dev/ttyUSB0

# Pixhawk via USB (alt):   /dev/ttyACM0

# If permission denied:

sudo usermod -aG dialout $USER   # then log out and back in

# 7  Debugging Reference

## 7.1  Startup Failures

| Symptom | Component | Likely Cause | Fix |
| --- | --- | --- | --- |
| FC connected via MAVSDK never prints | MOVE/MAVSDK | MAVProxy not running or wrong port | Start mavproxy.py first. Check --out port matches MAVSDK_ADDR. |
| GPS fix acquired never prints | MOVE/GPS | Drone indoors, no satellites | Take drone outside. Wait 60–90s. Check MAVProxy shows GPS lock. |
| Vision modules import error | INIT | 02_vision_filter.py etc missing from BASE_DIR | ls ~/safe_swarm/ — all numbered modules must be present. |
| C++ binary not found | CAMERA | bin/mlx_stdout not compiled | Run compile command from Section 6.2. |
| TCPCommandServer port in use | TCP | Previous run still alive | kill $(lsof -t -i :14560) then restart. |
| UDPSender init failed | UDP | DRONE_ID not in slave_1/slave_2/slave_3 | export DRONE_ID=slave_1 before running. |

## 7.2  Communication Failures

| Symptom | Where | Likely Cause | Fix |
| --- | --- | --- | --- |
| UDP send failed in slave logs | UDP | Master IP wrong or firewall | ping 10.42.0.1 from slave. Check MASTER_IP env var. |
| [slave_X] Connect failed in tcp_commander | TCP | Slave TCP server not started | Check main_orchestrator.py running. lsof -i :14560 on slave Pi. |
| ACK timeout for GOTO seq=N | TCP | Slave FC busy or MAVSDK frozen | Check slave logs for MOVE errors. Restart orchestrator if FC dead. |
| DroneTunnel send failed | Mine TCP | Master port 5000 not listening | Check udp_telemetry.py / app.py running on master. lsof -i :5000. |
| Telemetry stats: failed=N | UDP | Wi-Fi packet loss or wrong port | Check signal strength. iperf3 between Pi pair. Confirm port 14550 free. |
| No voice commands received | Voice | app.py not running or wrong URL | curl http://10.42.0.1/api/pending_commands — should return {"commands":[]} |

## 7.3  Flight Anomalies

| Symptom | Component | Likely Cause | Fix |
| --- | --- | --- | --- |
| Drone position always 0.0, 0.0 | Telemetry | BUG FIXED in v1.0: telem tasks never started | Ensure you have fixed _update_telemetry_loop (5 separate tasks). See Section 8. |
| Flight loop hangs at one waypoint | MOVE | BUG FIXED: no timeout in move_to_coordinate | Ensure 60s timeout is present. Drone will log WARN and continue. |
| Altitude critically low warning | SAFETY | Barometer drift or terrain | Normal if on-ground. Investigate if airborne below 0.3m. |
| Mine reported at wrong GPS | MINE | origin_state.json yaw wrong | Re-run 00_preflight_calib.py with drone pointing North. Recheck locked_yaw_rad. |
| Same mine reported multiple times | MINE | 1.5m dedup radius too small | Increase dedup radius in _handle_candidate if mines are close together. |
| Ghost mine detections | VISION | Low confidence threshold or no FPN | Check CONFIDENCE_THRESHOLD=0.80. Run preflight calib for FPN pattern. |
| Waypoint timeout after 60s | MOVE | GPS drift > 0.5m tol, or wind | Log is informational. Drone continues. Increase tol_m if GPS noisy. |

## 7.4  Quick Diagnostic Commands

| Command | What It Checks |
| --- | --- |
| lsof -i :14550 | See what process is using the UDP telemetry port |
| lsof -i :14560 | See what is using the TCP command port |
| lsof -i :5000 | See what is using the mine report port |
| arp -a | List all devices on the hotspot network (verify slave IPs) |
| ping 10.42.0.11 | Check slave_1 is reachable from master |
| ss -tulnp ¦ grep 14 | Show all open sockets on ports starting with 14 |
| cat logs/slave_1.log ¦ grep ERROR | Filter slave log for errors only |
| kill $(lsof -t -i :14560) | Force-kill whatever is holding port 14560 |
| python3 -c "import mavsdk" | Confirm MAVSDK installed correctly |
| ls -la bin/mlx_stdout | Confirm C++ binary compiled and has execute bit |

# 8  Known Bugs Fixed in v1.0

These bugs are documented here so you know exactly what to look for when debugging. All three are fixed in the current codebase.

Bug 1 — CRITICAL: Telemetry streams never started

File: main_orchestrator.py → MovementBlock._update_telemetry_loop()

Root cause: _ned, _hdg, and _flags tasks were defined and create_task()'d AFTER

the async for pos in drone.telemetry.position() loop. That loop is infinite —

it runs forever while connected. So _ned, _hdg, _armed, _landed tasks never started.

Symptom: drone position always showed 0.0, 0.0 on master. Heading/NED unavailable.

Fix: Each stream is now its own independent async def, all create_task()'d immediately

at the top of _update_telemetry_loop() before any loop runs.

Bug 2 — CRITICAL: Armed/LandedState streams mutually exclusive

File: main_orchestrator.py → MovementBlock._update_telemetry_loop()

Root cause: _flags() had two sequential async for loops — one for landed_state,

one for armed. The first loop (landed_state) is infinite. The second (armed) never ran.

Symptom: is_armed was always False. Safety monitor never showed armed=True.

Mission logic that checked is_armed would malfunction.

Fix: Each stream (_landed, _armed) is its own separate task. See Bug 1 fix.

Bug 3 — move_to_coordinate could hang forever

File: main_orchestrator.py → MovementBlock.move_to_coordinate()

Root cause: The while True arrival-check loop had no timeout. If GPS drift or wind

kept the drone 0.6m+ from the target permanently, the flight loop froze at that step.

All mission progress stopped. No timeout, no log, no recovery.

Symptom: Mission appears to freeze mid-flight. No further MINE or FLIGHT log lines.

Drone sits stationary indefinitely at its last commanded position.

Fix: MAX_WAIT_S = 60 timeout added. After 60s, logs WARN and breaks the loop.

Flight loop resumes from next step. Drone continues its strip scan.

## 8.1  How to Verify Fixes Are Present

# Check Bug 1+2 fix — look for 5 create_task lines before any async for loop:

grep -n 'create_task' main_orchestrator.py

# Should show: _pos, _ned, _hdg, _landed, _armed all created at top of function

# Check Bug 3 fix — look for MAX_WAIT_S constant:

grep -n 'MAX_WAIT_S' main_orchestrator.py

# Should show: MAX_WAIT_S = 60  and  if timeout_count >= MAX_WAIT_S * 5:

# 9  Competition Scoring Reference

ROBOFEST-GUJARAT 5.0 Senior Category. Total possible: 90 base + time bonus. Penalties unlimited.

| Task | Points & Notes |
| --- | --- |
| Safe takeoff + swarm activation | 10 pts — all drones arm and lift off cleanly on first attempt |
| Gesture/voice command recognition | 10 pts — hand gesture or voice triggers takeoff sequence |
| Mine detection & correct mapping | 30 pts — highest value task. Sensor must flag mine, GPS math must be correct |
| Safe path creation & marking | 20 pts — LED/powder/marker clearly shows 1m-clear corridor around mines |
| Person crosses safely | 20 pts — person walks field end-to-end following drone guidance |
| Time bonus | +5 pts/min saved — 10 min budget, every full minute saved adds 5 pts |
| Collision / unsafe proximity | −10 pts — per event. Triggered if drone enters 1m exclusion zone of mine |
| Drone crash / surface contact | −20 pts — per event. Any unintended contact with ground or obstacle |

Score maximisation strategy

Mine detection (30 pts) is the highest-value single task — prioritise sensor + GPS accuracy.

Time bonus: 10 min budget. If mission completes in 7 min → +15 pts bonus.

Penalty avoidance: one crash (-20) wipes out an entire bonus minute.

Safe path marking must be physically visible to judges — verify LED brightness outdoors.